Cybersecurity analysis of autonomous vehicles CAN networks using a CARLA–ROS virtual testbed

By: Marko Jovic

Abstract

This project builds a virtual testbed to study cybersecurity risks on the Controller Area Network (CAN) of autonomous vehicles using CARLA 0.9.13, ROS Noetic and PCAN usb adapter. The setup runs ROS on Ubuntu 20.04 in VirtualBox, connects to a CARLA server on Windows and integrates a PCAN-USB via SocketCAN to emulate real CAN traffic. I will evaluate attack techniques (replay, spoofing, and DoS), measure their effects on control and perception and prototype defence strategies, focusing on intrusion detection and control-loop safety.

Contents

1. Introduction	2

2. Background	2

3. System Architecture	3

4. Alternative Software and Hardware Options	3

4.1 Simulation Platforms	3

4.2 Middleware / Runtime	4

4.3 CAN Adapters	4

4.4 CAN Software and Tools	5

4.5 Virtualization / Orchestration	5

5. Methodology	6

5.1 Integration and Validation	6

5.2 PCAN-USB and SocketCAN	6

5.3 CAN Bridging	6

5.4 Attack Scenarios	6

5.5 Metrics	6

6. Implementation Details	7

7. Experiments	7

8. Discussion	7

9. Limitations	8

10. Future Work	8

11. Project Management	8

12. Summary and Conclusion	8

13. References	9

14. Bibliography	9

### 1. Introduction

Modern vehicles rely on CAN for inter-ECU communication but lack authentication and integrity protection, making them vulnerable to injection, spoofing, replay and DoS.

- This project creates a safe and reproducible simulation to:

  - Emulate control-plane behaviour, timing, and CAN framing.

  - Reproduce common CAN threats.

  - Prototype and evaluate detection/mitigation strategies without hardware risk.

Objectives

- Establish CARLA–ROS integration for ego-vehicle control.

- Emulate and transmit CAN traffic using PCAN-USB via SocketCAN.

- Implement and measure attack scenarios and their impacts on control and perception.

- Explore defence-in-depth with rate limiting, whitelisting and IDS.

Scope

- CARLA 0.9.13, ROS Noetic, Ubuntu 20.04 (VirtualBox), Python 3.7 (pyenv venv).

- CARLA–ROS networking over bridged LAN (Windows host-Ubuntu VM).

- PCAN-USB pass-through to VM with Linux SocketCAN and can-utils.

- Focus on control-plane security and timing.

## 2. Background

- CAN security lacks native authentication and encryption, arbitration prioritizes lower IDs, enabling DoS.

- Simulation-based evaluation using CARLA and ROS is common for control/perception studies, bridging to SocketCAN provides realistic timing and driver behaviour.

## 3. System Architecture

- Host (Windows 11): CARLA 0.9.13 server (ports 2000–2002).

- Guest (Ubuntu 20.04 VM, VirtualBox):

  - ROSNoetic, rosbridge (0.9.13branch), carla_twist_to_control, carla_ackermann_control.

  - Python 3.7 venv (pyenv), python-can, SocketCAN (can0), can-utils, socketcan_bridge.

- Hardware: PCAN-USB adapter passed through to VM (USB 2.0/3.0 filter).

- Data Flow:

  - CARLA server publishes simulation state and accepts control.

  - ROS bridge translates CARLA messages to ROS topics.

  - Control topics map to CAN frames via Python CAN node or socketcan_bridge.

  - PCAN-USB transmits on can0; monitoring via candump.

## 4. Alternative Software and Hardware Options

## 4.1 Simulation Platforms

- SVL (LGSVL)

  - Pros: Strong Autoware/Apollo integration, scenario editor, good sensor realism.

  - Cons: active development suspended in 2022.

  - Use case: Perception-heavy studies or Autoware/Apollo pipelines matter.

- AirSim (Microsoft)

  - Pros: Unreal-based, good sensor sim, easy Python/C++ control.

  - Cons: Weaker traffic vs CARLA, more focused on drones and AI research.

  - Use case: Control experiments with AI.

- BeamNG.tech (commercial)

  - Pros: Excellent vehicle dynamics and deformation.

  - Cons: High-end system required, ROS integration not as native.

  - Use case: If vehicle dynamics realism is the priority.

## 4.2 Middleware / Runtime

- ROS 2 (Humble)

  - Pros: DDS with QoS and SROS2 security, actively updated.

  - Cons: only supported on CARLA 9.14 and later.

  - Use case: If using newer versions of CARLA.

- Apollo (Cyber RT)

  - Pros: Specialized for Apollo AV stack, production-like pipelines.

  - Cons: Not officially supported for CARLA.

  - Use case: End-to-end AV security assessments.

- ZeroMQ

  - Pros: Lightweight pub/sub, low overhead.

  - Cons: Not officially supported for CARLA.

  - Use case: Embedded systems.

## 4.3 CAN Adapters

- Kvaser (Leaf Light, U100, etc.)

  - Pros: Robust timing, excellent drivers, SocketCAN and CANlib.

  - Cons: Costly, basic CAN tasks.

  - Use case: High-rate DoS realism and precise timestamps are required.

- Intrepid ValueCAN

  - Pros: Industry-grade, advanced features.

  - Cons: Cost, best in their ecosystem.

  - Use case: HIL and gateway testing with advanced logging/triggering, advanced analysis.

- Vector VN + CANoe/CANalyzer

  - Pros: Gold-standard diagnostics, scripting, conformance.

  - Cons: Expensive, windows-first; not SocketCAN-native.

  - Use case: OEM/Tier1 workflows, detailed protocol testing, advanced analysis.

- CANable/candleLight (open source)

  - Pros: Low cost, SocketCAN-native, great for research.

  - Cons: Smaller buffers, less robust, less tools.

  - Use case: Budget research, useful for smaller tests.

## 4.4 CAN Software and Tools

- Vector CANoe/CANalyzer (commercial)

  - Pros: Bus and virtual ECU simulation, diagnostics, scripting, security analysis.

  - Cons: High licenses cost, Windows focus with limited Linux support.

  - Use case: Test ECU communication, proof-of-concept development in vehicle networks.

- SavvyCAN

  - Pros: Open source, cross platform, reverse engineering features.

  - Cons: Less advance features and resources.

  - Use case: Free alternative, reverse engineering.

## 4.5 Virtualization / Orchestration

- Bare-metal Ubuntu

  - Pros: Best timing determinism for CAN and ROS.

  - Cons: Requires complex setup or second machine.

- Docker (privileged)

  - Pros: Simplifies access to host machine hardware.

  - Cons: USB/GPU pass-through complexity.

- WSL2

  - Pros: Development convenience.

  - Cons: Limited USB/CAN pass-through, networking issues.

## 5. Methodology

## 5.1 Integration and Validation

- Verify CARLA–ROS connectivity (bridge logs, rostopic list).

- Spawn and control ego vehicle (carla_spawn_objects, carla_twist_to_control).

- Validate timing and message rates (ROS topic frequencies).

## 5.2 PCAN-USB and SocketCAN

- Pass PCAN-USB to VM via VirtualBox USB filter.

- Load peak_usb and configure can0.

- Validate interface and traffic using ip link, candump.

## 5.3 CAN Bridging

- Two modes:

  - socketcan_bridge: ROS - SocketCAN topic-to-frame mapping.

  - Custom Python ROS node: Subscribes to CarlaEgoVehicleControl, maps throttle/steer/brake to CAN frames using python-can for protocol experiments.

## 5.4 Attack Scenarios

- Replay: Record legitimate frames, retransmit to bypass simple checks.

- Spoofing: Craft frames for critical IDs (e.g., steering, throttle) to simulate ECU impersonation.

- DoS: High-rate low-ID flooding to starve bus; evaluate arbitration impacts and control loss.

## 5.5 Metrics

- Control Stability: Path deviation, oscillation, settling time.

- Timing: Topic frequency, end-to-end latency (ROS - CAN - effect).

- Perception Degradation: Sensor latency, dropped frames (if sensor streams used).

- IDS Performance: True/false positive rates, detection latency, overhead.

## 6. Implementation Details

- CARLA–ROS bridge launched with version-aligned parameters and timeouts to ensure robustness.

- Vehicle control via ROS:

  - carla_twist_to_control for /twist inputs.

  - carla_ackermann_control for longitudinal/lateral control (PID tuned).

- CAN message design:

  - Example IDs for throttle/steer/brake.

  - Scaling and encoding aligned to 8-byte frames, timing via fixed or event-driven send rates.

- Safety guards:

  - Rate limiting outgoing CAN frames.

  - Whitelist allowed IDs for transmission.

  - Logging for reproducibility.

## 7. Experiments

- Baseline: Nominal driving with ROS control, measure steady-state latency and control metrics as a reference.

- Replay: Induces transient miscommands, measurable tracking error and short-lived instability.

- Spoofing: Sustained incorrect control leads to lane deviation and increased control effort, severity grows with spoof rate and priority.

- DoS: Low-ID floods cause control starvation, significant latency spikes and loss of actuation within tens to hundreds of milliseconds at high rates.

## 8. Discussion

- Security Implications: Middleware interfaces (CARLA–ROS) resemble real CAN weaknesses—unauthenticated control paths can be exploited if bridged directly to CAN.

- Control-Loop Sensitivity: Timing disruptions degrade stability quickly, arbitration favours DoS attackers using low IDs.

- Operational Trade-offs: Stricter defences may add latency, careful tuning preserves control quality.

## 9. Limitations

- Simulation ignores physical-layer nuances (bit timing, transceiver faults).

- CAN protocol mappings are simplified for experimentation.

- IDS strategies are lightweight and not cryptographic. Key management not addressed.

## 10. Future Work

- Integrate sensor topics and perception attacks (false object injection, etc.).

- Evaluate authenticated CAN variants or CAN-FD with MACs.

## 11. Project Management

- Milestones:

  - Setup CARLA–ROS (completed).

  - Manual control validation (completed).

  - PCAN-USB pass-through and SocketCAN config (ongoing).

  - Autonomous driving and creation of needed ROS nodes.

  - Attack scripts and measurement.

  - IDS prototyping and evaluation.

## 12. Summary and Conclusion

The testbed will demonstrate stable CARLA–ROS integration and a pathway to realistic CAN emulation using PCAN-USB. Attack experiments will highlight how injection, replay and DoS impact control stability and timing. Defence-in-depth rate limiting, whitelisting and anomaly-based IDS should improve resilience with acceptable overhead when properly tuned. The project should provide a reproducible foundation for CAN security research and might be extended to HIL and gateway validation.

## 13. References

CARLA Simulator Team (no date), CARLA Documentation (version 0.9.13). 
Available at: https://carla.readthedocs.io/en/0.9.13/
(Accessed: 19 September 2025).

CARLA ros-bridge (0.9.13) (no date), GitHub repository. 
Available at: https://github.com/carla-simulator/ros-bridge 
(Accessed: 19 September 2025).

ROS Noetic (no date), ROS Wiki. 
Available at: http://wiki.ros.org/noetic (Accessed: 19 September 2025).

The Linux Kernel Organization (no date), SocketCAN — Controller Area Network. Available at: https://docs.kernel.org/networking/can.html
(Accessed:  20 September 2025).

python-can (no date), Python-can documentation. 
Available at: https://python-can.readthedocs.io (Accessed: 20 September 2025).

can-utils (no date), GitHub repository. 
Available at: https://github.com/linux-can/can-utils (Accessed:  03 October 2025).

PEAK-System Technik GmbH (no date), PCAN-USB CAN USB Adapter — Product Page. Available at: https://www.peak-system.com/PCAN-USB.199.0.html?L=1
(Accessed: 03 October 2025).

‌

## 14. Bibliography

Apollo (no date), Cyber RT. 
Available at: https://developer.apollo.auto/cyber.html
(Accessed: 21 November 2025).

BeamNG (no date), BeamNG.tech. 
Available at: https://beamng.tech/ (Accessed: 21 November 2025).

Canonical (no date), Ubuntu Documentation. 
Available at: https://help.ubuntu.com/ (Accessed: 21 November 2025).

CANable (no date), CANable Interface. 
Available at: https://canable.io/ (Accessed: 23 November 2025).

Docker (no date), Docker Engine Documentation. 
Available at: https://docs.docker.com/engine/ (Accessed: 24 November 2025).

Intrepid Control Systems (no date), ValueCAN 4 Vehicle Network Adapter. 
Available at: https://intrepidcs.com/products/vehicle-network-adapters/valuecan-4/ 
(Accessed: 23 November 2025).

Kvaser (no date), Kvaser U100. 
Available at: https://kvaser.com/product/kvaser-u100/ (Accessed: 23 November 2025).

LGSVL (no date), SVL Simulator. Available at: https://github.com/lgsvl/simulator (Accessed: 22 November 2025).

Microsoft (no date), Windows Subsystem for Linux (WSL) Documentation. 
Available at: https://learn.microsoft.com/en-us/windows/wsl/ 
(Accessed: 24 September 2025).

Microsoft (no date), AirSim. Available at: https://microsoft.github.io/AirSim/ 
(Accessed: 22 November 2025).

ROS (no date), ROS 2 Humble Documentation. 
Available at: https://docs.ros.org/en/humble/index.html (Accessed: 19 September 2025).

SavvyCAN (no date), SavvyCAN. 
Available at: https://www.savvycan.com/ (Accessed: 22 November 2025).

Vector (no date), CANoe/CANalyzer Software. 
Available at: https://www.vector.com/int/en/products/products-a-z/software/canalyzer/ 
(Accessed: 25 November 2025).

Vector (no date), VN Interface Hardware Series. 
Available at: https://www.vector.com/us/en/products/products-a-z/hardware/network-interfaces/vn89xx/ (Accessed: 25 November 2025).

ZeroMQ (no date), ZeroMQ – Getting Started. 
Available at: https://zeromq.org/get-started/ (Accessed: 24 November 2025).